--------------------------------------------------------------------------------

--

--   AMD2910 Benchmark

--

-- Source:  AMD data book

--

-- author: Indraneel Ghosh

--         University Of California, Irvine, CA 92717

--

-- Written on Feb19, 1992

--

--------------------------------------------------------------------------------

OVERVIEW

The Am2910 is a microprogram address sequencer intended for use in high-speed

microprocessor applications.

Please see the block diagram of the Am2910 chip in the postscript file

"am2910.ps" while reading this description.

The Am2910 has a four-input multiplexer that is used to select either the

register/counter (R), direct data input (D), microprogram counter (uPC) or

the top of stack (TOS) as the source of the next microinstruction address.

The register/counter consists of 12 D-type flip-flops with a common clock.

When its load control, RLD_BAR is LOW, new data is loaded on a positive

clock edge. A few instructions include load or decrement. The output of the

register/counter, (R), is available to the multiplexer as a source for the

next microinstruction address. The direct input (D) furnishes a source of

data for loading the register/counter.

The microprogram counter, (uPC), is composed of a 12-bit incrementer followed

by a 12-bit register. The uPC can be used in either of two ways : When the

carry-in, CI, to the incrementer is HIGH, the microprogram register is loaded

on the next clock cycle with the current Y output word plus one

(Y + 1 --> uPC). Sequential microinstructions are thus executed. When CI is

LOW, the incrementer passes the Y output word unmodified so that uPC is

reloaded with the same Y word on the next clock cycle (Y --> uPC). The same

microinstruction is thus executed any number of times.

The third source for the multiplexer is the direct (D) input. This source is

used for branching.

The fourth source available at the multiplexer input is the top of the stack

(TOS). The stack is a 5-word by 12-bit register file. The stack is used to

provide return address linkage when executing microsubroutines or loops. The

stack contains a built-in stack pointer (SP) which always points to the last

file word written. This allows stack reference operations (looping) to be

performed without a pop.

The stack pointer operates as an up/down counter. During some micro-

-instructions, the PUSH operation may occur. This causes the stack pointer to

increment and the file to be written with the required return linkage. On

the cycle following the PUSH, the return data is at the new location pointed

to by the stack pointer.

During some microinstructions, a POP operation may occur. The stack pointer

decrements at the next rising clock edge following a POP, effectively

removing old information from the top of the stack.

The stack pointer linkage is such that any sequence of pushes, pops or stack

references can be achieved. At RESET (instruction 0), the depth of nesting

becomes zero. For each PUSH, the nesting depth increases by one; for each POP,

the depth decreases by one. The depth can grow to five. After a depth of five

is reached, FULL_BAR goes LOW. Any further PUSHes onto a full stack overwrite

information at the top of the stack, but leave the stack pointer unchanged.

This operation will usually destroy useful information and is normally

avoided. A POP from an empty stack may place non-meaningful data on the Y

outputs, but is otherwise safe. The stack pointer remains at zero whenever a

POP is attempted from a stack already empty.

The register/counter is operated during some microinstructions as a

12-bit down counter, with "R = 0" available as a microinstruction branch

test criterion. This provides efficient iteration of microinstructions. The

register/counter is arranged such that if it is preloaded with a number N

and then used as a loop termination counter, the sequence will be executed

exactly N + 1 times.

The device provides three-state Y outputs. These can be particularly useful

in designs requiring automatic checkout of the processor. The microprogram

sequencer outputs can be forced into high-impedance state, and pre-programmed

sequences of microinstructions can be executed via external access to the

address lines.

_______________________________________________________________________________

PORTS

It has the following ports :-

_______________________________________________________________________________

|  PORT    | TYPE | BIT   |                DESCRIPTION                        |

|	   |      | WIDTH |						      |

|==========|======|=======|===================================================|

| I	   |  in  |  4    | Four-bit instruction			      |

|__________|______|_______|___________________________________________________|

| CCEN_BAR |  in  |  1    | Conditional Code Enable bit			      |

|__________|______|_______|___________________________________________________|

| CC_BAR   |  in  |  1    | Conditional Code Bit			      |

|__________|______|_______|___________________________________________________|

| RLD_BAR  |  in  |  1    | Unconditional load bit for register/counter	      |

|__________|______|_______|___________________________________________________|

| CI       |  in  |  1    | Carry-in bit for microprogram counter	      |

|__________|______|_______|___________________________________________________|

| OEbar    |  in  |  1    | Tri-state driver control bit	              |

|__________|______|_______|___________________________________________________|

| clk      |  in  |  1    | Clock					      |

|__________|______|_______|___________________________________________________|

| D        |  in  | 12    | 12-bit data input to chip	                      |

|__________|______|_______|___________________________________________________|

| Y        | out  | 12    | 12-bit data output from chip		      |

|__________|______|_______|___________________________________________________|

| PL_BAR   | out  |  1    | Pipeline register output enable		      |

|__________|______|_______|___________________________________________________|

| VECT_BAR | out  |  1    | Vector output enable			      |

|__________|______|_______|___________________________________________________|

| MAP_BAR  | out  |  1    | Mapping PROM output enable 			      |

|__________|______|_______|___________________________________________________|

| FULL_BAR | out  |  1    | Stack full flag				      |

|__________|______|_______|___________________________________________________|

_______________________________________________________________________________

TABLE OF INSTRUCTIONS

The Am2910 has a 4-bit instruction. The actions performed in various modules

are controlled by the instruction as well as by the two condition code bits

(CCEN_BAR and CC_BAR) and by the contents of the register/counter.

The following table lists the actions performed by each module (e.g. stack,

register/counter etc.) according to the conditions listed above.

_______________________________________________________________________________

|       |                           |       ||         ||         |    |      |

| I3-I0 | NAME OF INSTRUCTION       |  REG. ||   FAIL  ||  PASS   |REG.|ENABLE|

|	|			    | CONT- ||_________||_________|CNTR|SIGNAL|

|	|			    | -ENTS ||   |     ||   |     |    |      |

|       |                           |       || Y |STACK|| Y |STACK|    |      |

===============================================================================

|       |			    |	    ||   |     ||   |     |    |      |

|    0  | JUMP TO ZERO (RESET)      |	X   || 0 |CLEAR|| 0 |CLEAR|HOLD|  PL  |

|_______|___________________________|_______||___|_____||___|_____|____|______|

|       | CONDITIONAL JUMP TO SUB-  |	    ||   |     ||   |     |    |      |

|    1  | -ROUTINE VIA PL (PIPELINE |	X   ||uPC| HOLD|| D | PUSH|HOLD|  PL  |

|       | REGISTER ADDRESS)         |	    ||   |     ||   |     |    |      |

|_______|___________________________|_______||___|_____||___|_____|____|______|

|       | JUMP VIA MAP ( ADDRESS    |	    ||   |     ||   |     |    |      |

|    2  | SUPPLIED BY MAPPING PROMS)|	X   || D | HOLD|| D | HOLD|HOLD|  MAP |

|_______|___________________________|_______||___|_____||___|_____|____|______|

|       | CONDITIONAL JUMP VIA PL   |       ||   |     ||   |     |    |      |

|    3  | (PIPELINE REGISTER ADDR.) |   X   ||uPC| HOLD|| D | HOLD|HOLD|  PL  |

|_______|___________________________|_______||___|_____||___|_____|____|______|

|       | PUSH STACK /CONDITIONAL   |       ||   |     ||   |     |    |      |

|    4  | LOAD REGISTER-COUNTER     |   X   ||uPC| PUSH||uPC| PUSH| ** |  PL  |

|_______|___________________________|_______||___|_____||___|_____|____|______|

|       | CONDITIONAL JUMP TO SUB-  |       ||   |     ||   |     |    |      |

|    5  | -ROUTINE VIA REG-CNTR. OR |   X   || R | PUSH|| D | PUSH|HOLD|  PL  |

|       | PL ADDRESS.               |       ||   |     ||   |     |    |      |

|_______|___________________________|_______||___|_____||___|_____|____|______|

|       | CONDITIONAL JUMP VIA      |       ||   |     ||   |     |    |      |

|    6  | VECT. (VECTOR ADDRESS)    |   X   ||uPC| HOLD|| D | HOLD|HOLD| VECT |

|_______|___________________________|_______||___|_____||___|_____|____|______|

|       | CONDITIONAL JUMP VIA      |       ||   |     ||   |     |    |      |

|    7  | REG-CNTR. OR PL ADDRESS   |   X   || R | HOLD|| D | HOLD|HOLD|  PL  |

|_______|___________________________|_______||___|_____||___|_____|____|______|

|       |                           |       ||   |     ||   |     |    |      |

|       | REPEAT LOOP IF            |  != 0 ||TOS| HOLD||TOS| HOLD| DEC|  PL  |

|    8  | REG-CNTR != 0             |_______||___|_____||___|_____|____|______|

|       | (ADDRESS FROM TOP OF      |       ||   |     ||   |     |    |      |

|       |  STACK)                   |  = 0  ||uPC| POP ||uPC| POP |HOLD|  PL  |

|_______|___________________________|_______||___|_____||___|_____|____|______|

|       |                           |       ||   |     ||   |     |    |      |

|       | REPEAT LOOP IF            |  != 0 || D | HOLD|| D | HOLD| DEC|  PL  |

|    9  | REG-CNTR != 0             |_______||___|_____||___|_____|____|______|

|       | (ADDRESS FROM PL )        |       ||   |     ||   |     |    |      |

|       |                           |  = 0  ||uPC| HOLD||uPC| HOLD|HOLD|  PL  |

|_______|___________________________|_______||___|_____||___|_____|____|______|

|       | CONDITIONAL RETURN FROM   |       ||   |     ||   |     |    |      |

|   10  | SUBROUTINE                |   X   ||uPC| HOLD||TOS| POP |HOLD|  PL  |

|_______|___________________________|_______||___|_____||___|_____|____|______|

|       | CONDITIONAL JUMP VIA PL   |       ||   |     ||   |     |    |      |

|   11  | ADDRESS AND POP           |   X   ||uPC| HOLD|| D | POP |HOLD|  PL  |

|_______|___________________________|_______||___|_____||___|_____|____|______|

|       | LOAD REG-CNTR AND         |       ||   |     ||   |     |    |      |

|   12  | CONTINUE                  |   X   ||uPC| HOLD||uPC| HOLD|LOAD|  PL  |

|_______|___________________________|_______||___|_____||___|_____|____|______|

|       |                           |       ||   |     ||   |     |    |      |

|   13  | TEST END OF LOOP          |   X   ||TOS| HOLD||uPC| POP |HOLD|  PL  |

|_______|___________________________|_______||___|_____||___|_____|____|______|

|       |                           |       ||   |     ||   |     |    |      |

|   14  | CONTINUE                  |   X   ||uPC| HOLD||uPC| HOLD|HOLD|  PL  |

|_______|___________________________|_______||___|_____||___|_____|____|______|

|       |                           |       ||   |     ||   |     |    |      |

|       |                           |  != 0 ||TOS| HOLD||uPC| POP | DEC|  PL  |

|   15  | THREE WAY BRANCH          |_______||___|_____||___|_____|____|______|

|       |                           |       ||   |     ||   |     |    |      |

|       |                           |  = 0  || D | POP ||uPC| POP |HOLD|  PL  |

|_______|___________________________|_______||___|_____||___|_____|____|______|

NOTE :-

** If FAIL then HOLD REG_CNTR else LOAD it.

"TOS" = Top Of Stack   	 	"Fail" = CCEN_BAR is 0 and CC_BAR is 1

"R" = Register-Counter		 	"Pass" = CCEN_BAR is 1 or CC_BAR is 0

"uPC" = Microprogram Counter	 	"!=" = Not Equal To

"I3-I0" = 4-bit Instruction  		"X" = Dont Care

_______________________________________________________________________________

OPERATION

The Table of Instructions shows the result of each instruction in controlling

the multiplexer which determines the Y outputs, and in controlling the three

enable signals PL_BAR, MAP_BAR and VECT_BAR.

The effect on the register/counter and the stack after the next positive

going edge is shown. The multiplexer determines which internal source drives

the Y outputs. The value loaded into the uPC is either identical to the Y

output, or else one greater, as determined by CI.

For each instruction, one and only one of the three outputs PL_BAR, MAP_BAR

and VECT_BAR is LOW.

Several inputs can modify instruction execution. In the ten conditional

instructions, the result is applied to CC_BAR. If the CC_BAR input is LOW,

the test is considered to have been passed, and the action specified in the

name occurs; otherwise, the test has failed and an alternate (often simply

the execution of the next sequential microinstruction) occurs. Testing of

CC_BAR may be disabled for a specific microinstruction by setting CCEN_BAR

HIGH, which unconditionally forces the action specified in the name; that is,

it forces a pass. Other ways of using CCEN_BAR include :

(1) tying it HIGH, which is useful if no microinstruction is data-dependent,

(2) tying it LOW if data-dependent instructions are never forced

unconditionally.

The effect of three instructions depends on the contents of the

register/counter. Unless the counter holds a value of zero, it is decremented;

if it does hold zero, it is held and a different microprogram next address is

selected. These instructions are useful for executing a microinstruction loop

a known number of times. Instruction is affected both by the external

condition code and the internal register/counter.

RLD_BAR, when LOW, causes the D input to be loaded into the register/counter,

overriding any HOLD or DEC operation specified in the instruction.

OE_BAR, normally LOW, may be forced HIGH to remove the Am2910A Y outputs from

a three-state bus.

The stack, a five-word last-in, first-out 12-bit memory, has a pointer which

addresses the last value presently on the top of the stack. Explicit control

of the stack pointer occurs during instruction 0 (RESET), which makes the

stack empty by resetting the SP to zero. After a RESET, and whenever else

the stack is empty, the contents of the top of stack are undefined until a

PUSH occurs. Any POPs performed while the stack is empty put undefined data

on the F outputs and leave the stack pointer at zero.

Any time the stack is full (five more PUSHes than POPs have occurred since the

stack was last empty), the FULL_BAR warning output occurs. This signal first

appears on the microcycle after a fifth PUSH. No additional PUSH should be

attempted onto a full stack; if tried, information within the stack will be

overwritten and lost.